SECTION 720	-	LANDSCAPE WORKS

##This section cross-references Sections 176 (or 177), 204 and 750.

If any of the above sections are relevant, they should be included in the specification.

If any of the above sections are not included in the specification, all references to those sections should be struck out, ensuring that the remaining text is still coherent:

720.01	GENERAL

This section covers the requirements for works associated with the vegetation of the site.  This includes, but is not limited to, material supply, site preparation, mulching, planting, grassing, irrigation and landscape maintenance as specified and shown on relevant Drawings, Planting Details and Schedules.

720.02	DEFINITIONS

(a)	Caliper

The stem or trunk diameter of a tree at a nominated point.  Caliper is measured at 300 mm above ground for sizing of nursery plants.

(b)	Individual Tree Planting

Any tree planting of single trees each surrounded by a mulch ring and/or grass cover.

(c)	Planting Area

Any continuous area where multiple plants are established using stock from containers such as pots, tubes and cells or by direct sowing (other than grassed areas).

(d)	Propagule

A propagule is any plant part capable of producing a new plant (e.g. seeds, cuttings, bulbs, corms, rhizomes, etc).

(e)	Tube stock or cell stock

Plants grown in small (less than 0.5 litre) containers.

(f)	Weed

Any plant which is not desired.  Sub-categories include:

(i)	Noxious weed.  Any plant listed as a weed for the project area under the Catchment and Land Protection (CALP) Act 1994.

(ii)	Species or individual plants specified to be removed.

(iii)	Weeds of planting beds and around individual trees include any plant not specified in the planting schedule or is not indigenous to the site.  This includes all local environmental weeds and noxious weeds.

(iv)	Weeds of mown-grass areas include all species listed in (i) above, broadleaf species (except broadleaf species specified for sowing) and other weeds where specifically listed.

© VicRoads  May 2012

Section 720 (Page 1 of 16)

720.03	SUBMISSIONS AND RECORDS

The specified inspections, samples, records and certificates shall be made available or submitted to the Superintendent for acceptance prior to the commencement of associated work.  Accepted samples shall become the agreed minimum standard and approved source of supply.  The Contractor shall be responsible for programming all inspections and review of samples and allow 2 working days notice to the Superintendent.  Samples, records and certificates shall be provided to the Superintendent at least 10 working days prior to bulk delivery.

***	The Contractor shall provide the following before commencement of the relevant landscape work: ##[strikethrough inapplicable items below]:

Inspections

	plant stock for all plant species

Samples

	mulch

	imported topsoil

Records

	evidence of origin of indigenous provenance plant material

	analysis certificates for grass seed

	details of suppliers

	soil tests on topsoil

	copies of fertiliser labels

720.04	SUPPLY OF MATERIALS

All materials shall be supplied by the Contractor unless otherwise noted.  If any materials are supplied by VicRoads, a joint inspection of the materials shall be made by the Contractor and the Superintendent following delivery of the materials to the site. The materials, if satisfactory, shall thereafter become the responsibility of the Contractor.

(a)	General Supply of Plant Stock

All plants shall be of the type, height, container size and trunk caliper specified in the Plant Schedules, Specification and Planting Plans.  These are minimum requirements.  No substitution of plants shall be allowed without the written agreement of the Superintendent.

Table 720.041  Plant Stock  Supply Properties

| Property | Requirement |
| --- | --- |
| General Requirements | All plants shall be hardened off for a minimum 4 weeks in conditions matching prevailing conditions on the Works site. |
| Species Integrity and Labelling | Plants shall be true to type and clearly labelled with at least one label per species per delivery batch. |
| Container Supply Sizes | Tubes shall be at least 0.2 litres in volume. Cells shall be at least 0.08 litres in volume. Other sizes shall be as specified. |
| Root Ball Occupancy | Roots shall be adequate to fill the container. Samples shall demonstrate that >85% of soil volume remains intact when the unsupported rootball is shaken. |

(Table continued next page)

© VicRoads  May 2012

Section 720 (Page 2 of 16)

Table 720.041  Plant Stock  Supply Properties  … continued

| Property | Requirement |
| --- | --- |
| Root Ball Integrity | Plants shall display a large, well developed and healthy fibrous root system with repeated and sequential division with no evidence of root curl, restriction or damage. The root ball shall not be pot bound and shall generally have roots penetrating to the edge of the pot. Except for monocots with fibrous root systems, plants with girdling, spiralling or j-rooting present in root systems will not be acceptable. |
| Uniformity of Plant Growth | The size shall be in proportion to the container size and species. Tube stock and cell stock shall not have shoot height greater than 3 times the depth of the container. |
| Caliper | Trees shall be self supporting. |
| Health and Vigour | Plants shall show vigorous tip growth and foliage size, texture and colour consistent with that shown in healthy specimens of the species. Plants shall have undamaged limbs and trunk. |
| Free of Pests and Diseases | Plants shall be free from pests and diseases at time of delivery, with visible attack from previously eliminated pests and disease restricted to <10% of foliage such that long term success of the plant is not affected. |
| Weed Free | Plants shall be provided in containers free of weeds. |

(b)	Supply of Indigenous Plant Stock

A species listed on the planting schedule shall be classed as an indigenous plant if the plant is likely to have occurred within 10 km of the project site (pre-1750).

The Contractor shall source seed or other propagules for indigenous plant stock from within or as close to the project area as practicable and from areas with similar soil type (same parent material) and rainfall (to minus 100 mm ARI).  Seed shall be collected in accordance with Florabank Guidelines and Codes of Practice (http://www.florabank.org.au/).  Further to these guidelines, seed shall not be collected more than 25 km from the project site without prior agreement with the Superintendent.  Grasses and wetland species may be sourced up to 50 km from the project site in areas with the same soil type and similar or lower annual rainfall.  Where a distinct form of a species occurs naturally in the local area, that form shall be used.

The Contractor shall be responsible for obtaining all necessary native flora collection permits from the Department of Sustainability and Environment.

The Contractor shall submit documentation of origin of indigenous provenance plant material which shall include the following details for each seed lot:

	species name

	storage code

	collection locality - Latitude/longitude or AMG

	approximate number of parent plants sampled

	approximate number of plants in that local population

All seed lots of the same species collected at different localities shall be supplied as a separate seed lots.  The provenance shall be tracked through propagation to planting for possible future seed collection purposes.

The format of the seed collection records shall be a spreadsheet compatible with Microsoft Excel.

© VicRoads  May 2012

Section 720 (Page 3 of 16)

(c)	Supply of Pesticides

Herbicides applied as a spray shall include sufficient coloured dye to identify application areas for at least 5 days.

(d)	Supply of Site Topsoil

Site topsoil shall be used wherever possible, in preference to imported topsoil.  Topsoil is defined as the weathered surface layer of soils that includes organic matter.

The Contractor shall be responsible for ensuring the growing medium supports normal healthy growth of the specified planting.  The Contractor shall determine when soil testing is required and shall undertake soil tests to establish what soil amendments and macro and micro nutrients are required to be added to the topsoil.

The Contractor shall notify the Superintendent of any topsoil characteristic which may reduce the performance of any plant species or grass mix.

(e)	Supply of Imported Topsoil

In the event that site topsoil is not available or is not adequate to achieve the specified depth of topsoil, the Contractor shall supply imported topsoil to achieve the total specified depth of topsoil.

Imported topsoil shall comply with the following properties:

Table 720.042  Imported Topsoil Blend Supply Properties

| Property | Requirement |
| --- | --- |
| General Requirements | Imported topsoil shall be suitable for supporting healthy plant growth of the specified and scheduled species. |
| Texture | The texture shall either match the site topsoil texture or provide a light to medium friable sandy loam with a clay content 10-15%, capable of handling when moist but lacking cohesion so that it will fall apart easily. |
| Extraneous Material and Contaminants | The topsoil shall be free of sods of subsoil, rubbish, petrol and oil contaminants, lime etc, and meet the requirements for clean fill in EPA publication 448-Classification of Wastes as ‘Fill Material’ and shall have contaminant levels less than the concentration listed in Table 2 of that document. Stones shall be <5% (by dry weight) with stone size not exceeding 25 mm. |
| Dry Bulk Density (kg/m3) | Bulk density shall be >1000. |
| Soil Nutrients | Adequate levels to support normal plant growth for the scheduled species. |
| Organic Matter (% by mass) | Decomposed matter shall be 3-15% by mass, and undecomposed matter shall be <5% by mass. |
| Weed Content and Pathogens | Soil shall meet AS 4419 5.4. No noxious weeds, noxious seed or Phytophthora cinnamomi. |

(Table continued next page)

© VicRoads  May 2012

Section 720 (Page 4 of 16)

Table 720.042  Imported Topsoil Blend Supply Properties  … continued

| Property | Requirement |
| --- | --- |
| Wettability (mm/min) | Soil shall meet AS 4419 5.5 |
| Soil pH | 5 – 8 |
| Phosphorous Content (mg/kg Olsen method) | 5-10 |
| Electrical Conductivity (dS/m) | Soil shall meet AS 4419 5.6 |
| Dispersibility Category | Soil shall meet AS 4419 5.11 |
| Nitrogen Drawdown (NDI150) | Soil shall meet AS 4419 5.12 |
| Permeability | Soil shall meet AS 4419 5.13 |
| Toxicity | Soil shall meet AS 3743 |

(f)	Supply of Fertilisers

Fertilisers for newly grassed areas shall be N:P:K 10:4:6 with trace elements.

Fertilisers for planting areas and advanced trees shall be slow release with a 9-12 month release period and suitable for the establishment of plant types, sizes and species specified in the plant schedule(s).

(g)	Supply of Gypsum

Gypsum (Calcium Sulphate) shall comply with the minimum specifications as laid out in the current Victorian Fertiliser Regulations.  Only manufactured gypsum shall be used.  Gypsum derived from recycled plaster board shall not be used.  Sodium shall not exceed 1% by mass.

(h)	Supply of Wood Mulch (Chipped or Shredded)

All wood mulch shall be supplied, delivered, handled and applied in accordance with AS 4454.

(i)	Imported Wood Mulch

Wood mulch shall comply with the following properties:

Table 720.044  Shredded or Chipped Wood Mulch General Supply Properties

| Property | Requirement |
| --- | --- |
| General Requirements | Mulch shall meet AS 4454, Table 3.1 for composted or pasteurised mulch. Mulch type shall have fibrous properties which 'interlock' the particles so that installed mulch is not blown away by wind. |
| Purity | Mulch shall be free of soil and weeds, plant pathogens, vermin and toxins. Plywood, composite timber product and painted timber shall not exceed a combined maximum of 1% by weight total. Mulch shall be free of treated pine, exposed sharp metal, plastic and other litter. |
| Sizing | Meet AS 4454–2012, Table 3.1(A) In planting areas adjacent to either roads posted at <80 km/h, pedestrian or cycle paths or recreational open space, 95% of particle size shall be no wider than 40 mm and no longer than 140 mm. No particles longer than 200 mm. In planting areas adjacent to roads posted at ≥80 km/h, 80% of particle size shall be no wider than 50 mm and no longer than 140 mm. No particles longer than 400 mm. |

© VicRoads  May 2012

Section 720 (Page 5 of 16)

(ii)	Site Mulch

Site mulch available from the chipping of on-site indigenous or non-weed vegetation shall be used where possible.  Any species that could have weed potential by seed or vegetative fragments shall not be recycled in the landscape works.  Mulch shall satisfy the purity and sizing requirements shown in Table 720.044.

(i)	Supply of Weed Control Mat and Erosion Control Mat

The Contractor shall supply and install jute or approved equivalent, biodegradable, fire retardant weed and erosion control mat as indicated on the Drawings or as specified.  Weed Control Mat shall be minimum 700 grams per square metre and Erosion Control Mat shall be 300 grams per square metre.  The surface of the mat shall permit water infiltration.

(j)	Supply of Tree Guards, Stakes and Marker Stakes

Tree guards shall be either opaque plastic and of sufficient gauge and UV inhibitor to ensure a minimum two year life or minimum 2 litre cartons of sufficient construction to ensure a minimum 18 month life.  Tree guard stakes and marker stakes shall be sufficiently robust to be driven into the ground, securely support the tree guards and ensure a minimum two year life.

(k)	Supply of Grass Seed

Grass seed shall comply with the following properties:

Table 720.045  Grass Seed Properties

| Property | Requirement |
| --- | --- |
| Species and Proportion | Areas with greater than 700 mm average annual rainfall • Calypso II Fine Leaf Perennial Ryegrass 20% by count • Roper Perennial Ryegrass 50% by count • Creeping Red or Chewings Fescue 20% by count • Kentucky Bluegrass 10% by count Areas with less than 700 mm average annual rainfall • Camel Perennial Ryegrass 25% by count • Guard or Safeguard Annual Ryegrass 35% by count (or another toxicity resistant variety of L. rigidum) • Currie or Porto Cocksfoot 15% by count • Trikkala Subterranean Clover 25% by count |
| Minimum Seed Purity | 98% and shall not contain noxious weed seed |
| Minimum Germination Rate | 90% at date of supply |

The quality and specified properties of all seed shall be analysed and certified by either a NATA (National Association of Testing Authorities) or ISTA (International Seed Testing Authority) laboratory.  Labelling of all seed shall comply with the Australian Seed Federation National Code of Practice for Seed Labelling and Marketing.

(l)	Supply of Sterile Grass Seed

Sterile Rye-corn (Secale cereale) seed shall be used for the stabilisation of areas to be subsequently sown or planted and for stabilisation of temporary batters and stockpiles.

© VicRoads  May 2012

Section 720 (Page 6 of 16)

720.05	PEST AND WEED CONTROL

(a)	Pest and Weed Management Procedures

The Contractor shall develop and implement site specific pest and weed management procedures to control pest and weed populations.  The procedures shall consider pest animals, plants, fungi and other pathogens that may impede plant establishment.

Weed management procedures shall include removal of weeds from their existing locations before, during and after site clearing, topsoil stripping and landscape works.  Weed management procedures shall minimise the spread of weeds in topsoil to other locations within and outside the site.

In addition to Sections 204 and ##176 (or 177): of the VicRoads Standard Specifications for Roadworks and Bridgeworks, pest and weed management procedures shall include control of soil borne pathogens including, but not limited to Phytophthora cinnamomi, Armillaria sp. and soil borne weeds.

(b)	General Requirements

Pesticides shall be used in accordance with the manufacturer’s instructions.  Where off-label use of pesticides is considered desirable to achieve pest control requirements, the Contractor shall be responsible for the investigation and procurement of off-label permits.  The Contractor shall advise the Superintendent of any proposed off-label pesticide use prior to pesticide application.

Weed and pest control personnel shall be qualified and experienced in the use of pesticides, control of weeds, prevention of pesticide drift and identification of target and non target species found on site and included in the planting schedules.

The Contractor shall prevent runoff into waterways of pesticides not registered for use in waterways.

If ‘non-target’ areas or plants are damaged during weed control works, then the Contractor shall be responsible for the reinstatement of those areas to the satisfaction of the Superintendent.

Records of herbicide use shall be made available to the Superintendent upon request.

(c)	Pre-Planting Weed Control

Pre-planting weed control shall be undertaken to ensure that planting areas and new grassed areas are free of visible weeds prior to the commencement of planting and grassing.  Weed control shall be programmed and implemented to deplete the potential weed seed bank within the topsoil to be used in planting beds.

720.06	SETTING OUT OF LANDSCAPE SOFTWORKS

The Contractor shall set out the location and shape of planting areas in accordance with the Landscape Drawings.

The Contractor shall comply with the plant density requirements and planting offsets as shown in the plant schedule(s).

Individual plants shall be located in accordance with the drawings such that the mature form of the plant will be in accordance with VicRoads Road Design Guidelines, including clear zone and sight distance requirements.

© VicRoads  May 2012

Section 720 (Page 7 of 16)

720.07	SITE PREPARATION

Further to the requirements of Section 204, the Contractor shall provide the soil preparation described in Table 720.071.  All elements of soil preparation proceed in order from left to right.

Table 720.071  Soil Preparation Requirements

| Situation | Initial Ripping of Subsoil Prior to Topsoil Placement | Minimum Topsoil Depth | Cross Ripping | Cultivation |
| --- | --- | --- | --- | --- |
| Planting Areas in disturbed areas flatter than 3h:1v | Min 400 mm deep at max 500 mm spacing. Leave all batters with roughened subsoil to assist with keying of topsoil. | 150 mm | - | Min 150 mm deep |
| Planting Areas in disturbed areas steeper than 3h:1v | Min 300 mm deep at max 500 mm spacing, between parallel to and 45° from contour. Leave all batters with roughened subsoil to assist with keying of topsoil. | 100 mm | - | - |
| Planting Areas in undisturbed areas | Min 400 mm deep at max 500 mm spacing | - | - | Min 150 mm deep |
| Grassing in disturbed areas | Min 200 mm deep, 500 mm max spacing | 75 mm, except 100 mm in swales | Min 100 mm deep, 500 mm max spacing | - |
| Grassing in undisturbed areas | - | - | - | Min 100 mm deep |
| Individual Tree Planting (including advanced trees) within grassed areas. The area of treatment is 25 m2 per tree. | Min 400 mm deep at 500 mm max spacing | 150 mm in disturbed grassed areas | Min 300 mm deep, max 500 mm spacing | - |
| Notes: • Disturbed areas include any area of cut batter, fill batter, site of excavation (including borrow pit), site of dumping (including fill mound), drains, swales or area driven over by machinery resulting in soil compaction. Disturbed areas also include any areas where topsoil is effectively absent or areas of soil with dry bulk density exceeding the maximum shown in Table 720.072. • Undisturbed areas are those areas with the pre construction soil profile retained intact, without compaction or soil loss. • Initial ripping in granitic soils should be parallel to the contour. • Topsoil on grassed verge areas shall be left firm. |  |  |  |  |

(a)	Soil Additives including Gypsum

Soil additives, including any material used to modify the chemical properties of the soil, shall be applied by the Contractor prior to physical ripping and cultivation and in accordance with results and recommendations from site topsoil analysis tests.

The Contractor shall supply and install gypsum to the subsoil where subsoils are dispersive (AS 4419 5.11).  The Contractor shall apply sufficient gypsum to ensure soils are not dispersive, incorporating gypsum during ripping to a minimum depth of 300 mm.

The Contractor shall adjust the soil pH if the soil, including subsoil, to a depth of 300 mm is not in the range of 5-8.

Where salinity requirements for existing (unmoved) site subsoil or topsoil do not meet the acceptable range in AS 4419 5.6, the Contractor shall notify the Superintendent.

© VicRoads  May 2012

Section 720 (Page 8 of 16)

(b)	Ripping

The Contractor shall undertake any ripping specified.  Ripping shall occur when soil is at or approaching the plastic limit of dryness.  Ripping shall be postponed when the soil is at or wetter than field capacity.  Narrow tines shall be used in rocky ground and winged tines in other ground types.  Rock and rubbish brought to the surface shall be removed and disposed.  The Contractor shall inform the Superintendent of sites where the presence of rock prevents the specified minimum ripping depth being achieved and implement alternative techniques (such as a standard tine) to maximise planting bed ground preparation depth.

Ripping shall not occur within the extent of existing vegetation areas marked for protection or within the drip-line of existing woody plants.  In the vicinity of woody plants to be retained with greater than 600 mm trunk diameter at breast height, 1.5 times the height of the tree measured horizontally from the tree trunk shall be considered and marked as an area to be protected.  For trees less than 600 mm diameter the area to be protected shall be the area within the drip-line.  Any planting to be carried out within these protected areas shall be done using hand cultivation techniques to minimise damage to existing tree roots zones.

Where existing planting beds are to be extended, the unplanted areas shall be ripped as described above.

HP	The Contractor shall make available each ripped planting area, individual tree planting and grassing area prior to mulching, planting or grassing.  The Contractor shall not proceed until the Superintendent acknowledges requirements for ripping have been met.  In areas where ripping is not practicable, the Contractor shall submit alternative methods to achieve an equivalent result to the Superintendent for review.

(c)	Topsoil Application

Following subsoil preparation, topsoil shall be spread on planting areas and grassed areas.  Topsoil shall not be compacted.

Weedy site topsoil shall not be spread to other locations on the site.  Weedy topsoil shall be handled in accordance with the Contractor’s weed management procedures.

(d)	Cultivation

Cultivation shall be undertaken to break soil down to a tilth with 80% of the volume composed of clods less than 50 mm in diameter.  Cultivation equipment that might create a hard pan beneath the depth of cultivation (such as rotary hoes and other powered cultivators) shall not be used.  In the event of saturated ground conditions, cultivation shall be delayed until the ground has dried out to a suitable condition.  Where ripping and topsoiling is sufficient to allow easy manual excavation of soil to a depth of 300 mm or more, cultivation may be omitted.

Cultivation shall not occur within the extent of existing vegetation areas marked for protection or the drip-line of existing woody plants.  Hand cultivation shall be used within root zones of trees to be retained.

(e)	Topsoil Finished Surface Levels

After cultivation and trimming, the Contractor may lightly compress topsoil for stability on slopes.  Vibrating plates shall not be used.  The Contractor shall ensure dry bulk density does not exceed the following:

© VicRoads  May 2012

Section 720 (Page 9 of 16)

Table720.072  Maximum Dry Bulk Density of Soils for Planting and Grassing

| Soil Texture (as determined by AS 4419) | Maximum Dry Bulk Density for Topsoils |
| --- | --- |
| Clay soils | 1.3 g/cm3 (1.4 g/cm3 on slopes steeper than 3:1) |
| Loams | 1.5 g/cm3 |
| Sandy soils | 1.7 g/cm3 |

The finished level of grassed areas shall be:

(i)	not higher than 125 mm below weep-holes or damp-proof courses in adjacent walls;

(ii)	level with the bottom of fence plinths;

(iii)	level with adjacent pavements, kerbs, pits and other structures.

The finished level of garden bed mulch and/or topsoil shall be tapered so that the finished mulch and/or topsoil levels meet flush with the adjacent surfaces.

The Contractor shall ensure these conditions are maintained and there is no compaction of prepared soils before planting.

HP	The Contractor shall make available each ripped, topsoiled and cultivated planting area, individual tree planting and grassing area prior to mulching, planting or grassing.  The Contractor shall not proceed until the Superintendent acknowledges requirements for topsoiling and cultivation have been met.  In areas where topsoiling or cultivation is not practicable, the Contractor shall submit alternative methods to achieve an equivalent result to the Superintendent for review.

(f)	Removal of Unwanted Matter

The Contractor shall remove unwanted matter.  Unwanted matter includes the following:

Table 720.073  Unwanted Matter

| Mown Grass Areas | Planting Areas |
| --- | --- |
| • any weeds, plastic, metal, glass or material toxic to plants • surface stone, rock, building rubble greater than 25 mm in diameter • clay lumps, sticks and exposed tree roots greater than 50 mm diameter | • any weeds, plastic, metal, glass or material toxic to plants • any visible surface stone, rock, building rubble greater than 75 mm in diameter • clay lumps, sticks and exposed tree roots greater than 100 mm diameter |

(g)	Existing Planting Areas

Existing planting areas shall exclude areas of naturally occurring remnant vegetation.

Prior to undertaking site preparation or planting within existing plantations, all dead, severely damaged (<40% intact canopy) and fallen trees and shrubs shall be removed from the plantation.  Woody weeds with any seed or vegetative material capable of regeneration are to be removed and disposed of-off-site.

Any severely pest affected vegetation that cannot be successfully treated using pesticides (or any other treatment) shall be removed and disposed of off-site.  All remaining vegetation within the plantation is to be retained and protected from direct and indirect damage from ground preparation, weeding, mulching and infill planting.

© VicRoads  May 2012

Section 720 (Page 10 of 16)

Remaining vegetation in the existing plantation shall be assessed for damaged, dead or diseased limbs and branches.  All such limbs and branches on existing vegetation with a diameter greater than 50 mm in urban areas and 100 mm in rural areas are to be removed.  All pruning works including dead-wooding, canopy uplifting and weight reduction shall comply with the requirements of AS 4373.

The Contractor shall obtain all necessary permits and obtain approval from the Superintendent prior to the pruning or removal of local native vegetation.

720.08	MULCHING

(a)	Wood Mulch (Shredded or Chipped)

The Contractor shall install wood mulch to all Planting Areas unless otherwise specified.  Mulch shall extend at least 1 metre beyond plant centres at the outer edges of all Planting Areas.  If space does not permit this in median, separator or splitter-island planting, mulching shall extend to the back of kerb.

The Contractor shall install 2 metre diameter mulch rings around all Individual Tree Plantings.

Mulch depths in all situations shall be a minimum 75 mm and maximum 125 mm.

Mulch shall be raked to a even, neat appearance and kept clear of plant stems to avoid collar rot.

(b)	Weed Control Mat and Erosion Control Mat

The Contractor shall install weed control mat for planting areas steeper than 2:1 and in planting areas prone to flooding or within 2 m of waterways, swales, wetlands and sedimentation ponds or elsewhere as specified.

Weed control mat and erosion control mat shall be laid and anchored in accordance with the manufacturer’s instructions.

720.09	PLANTING

(a)	Planting

The Contractor shall carry out planting so as to ensure healthy, vigorous growth of plants. In the event the specified plant is unlikely to be suited to the as-constructed growing conditions, the Superintendent shall be notified.

Holes in heavy soils or on batters shall be prepared so as to ensure adequate drainage.

Holes shall not be left smooth sided in basalt or ‘plastic’ soils.  Drill or auger hole diggers shall only be permitted where soils have been cultivated.

Planting holes shall be backfilled with friable topsoil free of debris, rocks and clods greater than 50 mm in diameter.

(b)	Fertilising

The Contractor shall fertilise advanced trees and, where specified and/or identified by the Contractor’s soil analyses, other planting areas with fertiliser in accordance with the manufacturer’s recommendations.

© VicRoads  May 2012

Section 720 (Page 11 of 16)

(c)	Initial Watering

The Contractor shall saturate each plant within 8 hours after planting.  Cells and tubes shall be irrigated with a minimum 3 litres of water per plant.  Other container sizes shall be irrigated with a volume of water greater than the container size.

Under no circumstances shall any plant be planted into a dry planting hole (soil moisture at wilting point or drier).  If the soil is dry, the planting hole shall be saturated with water prior to planting.

720.10	GRASSING

(a)	Seeded Grassing

The Contractor shall seed grass to:

•	all areas indicated to be grassed on the Drawings, and

•	all areas disturbed by the Contractor which will not be a planting area or managed as remnant indigenous vegetation.

Grass seed shall be sown at minimum 100 kg/Ha (if drilled) or 200 kg/Ha (if broadcast), distributed evenly to achieve an even and dense grass cover.

Germination rate shall be minimum 80% cover within 8 weeks and 95% cover within 3 months of sowing.  If germination has not been achieved in any grassed area within 8 weeks (except during December to March), then the area(s) shall be reseeded with the specified grass seed mix.

(b)	Fertilising

The Contractor shall apply fertiliser evenly over the prepared surface in accordance with the manufacturer’s recommendations.

(c)	First Mow

The Contractor shall carry out the first cut when at least 50% of the grassing area to be cut has grown to minimum 75 mm and maximum 150 mm height.  Mow to a minimum height of 75 mm.  Less than 1/3 the height of the grass shall be removed in the first cut.

720.11	LANDSCAPE MAINTENANCE

(a)	Scope of Maintenance

Landscape maintenance tasks shall begin prior to Practical Completion as specified in Clause 720.11(d) and shall continue until the completion of the Defects Liability Period for the Whole of the Works or the Defects Liability Period of the landscape works, whichever is the latter.

***	Maintenance of the landscape work shall include the following tasks:

(i)	replanting

(ii)	weed control

(iii)	watering

(iv)	mowing/slashing

(v)	reseeding of seeded grass areas

(vi)	pest and disease control

(vii)	re-mulching

© VicRoads  May 2012

Section 720 (Page 12 of 16)

(viii)	pruning

(ix)	maintaining the site in a neat and tidy condition

(x)	repair and removal of tree guards and stakes

(xi)	repairs to erosion affected areas

(xii)	stockpile areas.

(b)	Maintenance Program and Joint Inspections

***	The Contractor shall prepare and submit a maintenance program showing sufficient information to enable the landscape maintenance works to be evaluated and shall show as a minimum the following:

•	inspection visits during the period of maintenance;

•	maintenance works during the period of maintenance including an outline of replacement planting regime and proposed pest and weed management activities.

Quarterly joint inspections shall be undertaken each year by the Contractor and the Superintendent after commencement of the Defects Liability Period.

Any remedial work shall be performed within three weeks of the date of inspection.  Grassing and planting may be delayed until suitable conditions prevail subject to the agreement of the Superintendent.

(c)	Plant Performance Requirements

Shrub and groundcover planting bed areas shall achieve the following performance requirements:

***	Table 720.111	Performance Requirements for Shrub and Groundcover Planting Areas

| 12 months after Practical Completion | 24 months after Practical Completion |
| --- | --- |
| • minimum ##:25% ground closure | • minimum ##:60% ground closure |
| • all plants showing healthy growth | • all plants showing healthy growth |
| • performance indicating probable ##:90% ground closure at 36 months after Practical Completion | • performance indicating probable ##:90% ground closure at 36 months after Practical Completion |

Planting that does not meet the above performance requirements shall be re-mulched and replanted annually to 100% of the original planting density for the area of planting area gaps.

Tree planting areas shall achieve the following performance requirements:

Table 720.112  Performance Requirements for Tree Planting

| Previous 12 months rainfall | Minimum Growth Rate Tubes/Cells | Minimum Growth Rate Advanced Trees |
| --- | --- | --- |
| More than 700 mm | 750 mm per annum | 300 mm per annum |
| 500 – 700 mm | 500 mm per annum | 200 mm per annum |
| Less than 500 mm | 400 mm per annum | 100 mm per annum |

© VicRoads  May 2012

Section 720 (Page 13 of 16)

Tree planting that does not meet the above performance requirements shall be replaced annually to achieve 100% of the original specified planting numbers.

Planting that does not meet the performance requirements because of soil conditions, the site shall be re-prepared and replanted at the Contractor’s expense.

(d)	Maintenance Tasks

The Contractor shall maintain the works as follows in accordance with the requirements of this specification, the drawings and plant schedules.

(i)	Noxious Weeds and Weeds to be Removed

Eradication of noxious weeds and weeds to be removed over the whole site shall commence from the date of Possession of Site.  The weed eradication targets shall be achieved within six months of Possession of Site and then shall be maintained.

***			The Contractor shall eradicate:

•	all state prohibited weeds

•	all regionally prohibited weeds

•	all regionally controlled weeds

•	the following Weeds of National Significance (WONS):

-	## (specify):

•	the following priority weeds identified in the local Catchment Management Authority Weed Action Plan:

-	## (specify):

(ii)	Mown Grass Weed Control

Weeds within grass areas are to be managed from cultivation of new grass areas.

Mown grass shall be maintained to control broad leaf herbaceous and woody weed populations.  Weed control shall be implemented to control broad-leaf weeds that exceed 10% cover in urban areas or 20% over in rural areas. Weed control shall occur prior to the weed plant reaching flowering.

Areas of native broadleaf cover among mown grass shall be brought to the attention of the Superintendent prior to spraying.  The Superintendent will direct whether these areas shall be treated.

Bare ground that results from weed control activities shall be cultivated and re-grassed to establish a minimum 95% grass cover.

(iii)	Weed Control in Planting Areas and Individual Tree Plantings

Weeds within planting areas and around tree plantings are to be managed from when cultivation of the planting site is complete.

All planting areas shall be managed by the Contractor to maintain a minimum of 90% weed free surface with no weeds taller or broader than 200 mm at all times and all weeds are to be removed prior to the production of viable seed.  Weeds within Individual Tree Plantings are to be managed for a radius of not less than 1.5 metres at each planting location.

(iv)	Pest Animals and Diseases

The control of pests and diseases shall begin before the first planting.  The Contractor shall maintain all plants free of insect infestation and plant disease so that healthy, vigorous plant growth is sustained.  Minor infestations of native pests and diseases on indigenous plants not threatening survival or healthy growth need not be controlled.

© VicRoads  May 2012

Section 720 (Page 14 of 16)

The Contractor shall provide protection to plants from vertebrate grazing animals.  All planting killed by grazing animals shall be replaced.  If more than 5% of planting in a planting area shows signs of damage by grazing animals, the Contractor shall take immediate action to prevent further grazing damage.  As a minimum the Contractor shall immediately install tree guards or suitable protective fencing securely around all plants subject to vertebrate grazing.

Where chemical repellents are to be used to prevent grazing, the repellent shall be applied until the growing tip of the plant is beyond the grazing height of the pest animal, subject to the approval of the Superintendent.

(v)	Replanting

Maintenance for replanting shall begin as soon as planting has begun in a planting area.

Plants which die or do not achieve the specified growth performance shall be replaced and replanted at the Contractor’s expense.

The Contractor shall program and allow for the propagation and supply of plants for any replanting during the current or next available planting season.  A full pass of replanting shall occur at least once in each year of the Defects Liability Period. Replanting shall be undertaken either in Autumn during the months of April or May or Spring during the months of September or October.  Replanting outside of these times can be undertaken with approval from the Superintendent.

(vi)	Watering

The Contractor shall water all plants as necessary to ensure continued healthy and vigorous growth from time of planting.

(vii)	Grass Mowing ##(specification writer to strikethrough one of the following and delete this comment):

***			Mowing of all grass areas shall commence from Possession of Site.

***			Mowing of all grass areas shall commence from the earlier of sowing or road opening to traffic.

In addition to areas grassed by the Contractor, the Contractor shall maintain all other grassed areas within the limits of work in accordance with Routine Maintenance Specification RM511 – Grass Mowing, Section 750 of Standard Specification for Roadworks and Bridgeworks.

Areas with less than 95% grass cover (for each grassed area) within three months after sowing shall be re-sown by the Contractor.  The Contractor shall ensure that 95% cover is maintained throughout the duration of the maintenance period.

(viii)	Re-mulching

Maintenance of areas mulched shall begin from the first planting.

Areas mulched with shredded wood mulch that do not have full foliage cover at ground level shall be maintained at a minimum consolidated depth of 75 mm by the Contractor.  Re-mulching is not required where shrub or groundcover canopies have fully connected foliage.

© VicRoads  May 2012

Section 720 (Page 15 of 16)

(ix)	Pruning

Pruning maintenance shall begin from the first planting.

The Contractor shall prune trees and shrubs to remove damaged limbs and branches with a diameter above 10 mm. Tree branches likely to form a dominant ‘U’ or ‘V’ shaped crotch shall be pruned to a single leader.  All pruning and removal of living or dead stems greater than 10 mm diameter shall be in accordance with AS 4373 Pruning of Amenity Trees.

(x)	Maintaining the Site in a Neat and Tidy Condition

At any time the site is accessible to the public, the Contractor shall keep the site in a neat and tidy condition free of litter, debris or extraneous materials not associated with the works.  Litter that may be carried from the site by wind or water shall be controlled, collected and removed by the Contractor from Possession of Site.

(xi)	Maintenance of Tree Guards and Stakes

Maintenance of tree guards and stakes shall begin from installation.

Tree guards shall be maintained by the Contractor to ensure healthy vigorous plant growth is not inhibited.  Tree guards shall be removed when the plant reaches 750-1000 mm in height.  Tree guards shall be removed prior to Final Completion of the Contract unless otherwise directed by the Superintendent.

Marker stakes shall be removed when the plant reaches the height of the stake.

(xii)	Repairs to Erosion Treated and Affected Areas

Maintenance and repairs to erosion treated and affected areas shall begin from the earlier of installation of treatment or erosion occurring.

The Contractor shall maintain all areas of erosion protection treatments from the earlier of installation of treatment or erosion occurring and shall repair all damage or erosion which arises.  Such areas shall be re-prepared and re-protected as necessary to minimize erosion occurring and to establish a stable condition.

(xiii)	Intervention Standards

***			In addition to the above requirements, the Contractor shall maintain the landscaping works in accordance with Routine Maintenance Category Level  ##1: of the Intervention and Rectification Standards specified in Section 750 of VicRoads Standard Specification for Roadworks and Bridgeworks for:

•	RM411 – Surface Drains and Verges

•	RM511 – Grass Mowing

•	RM512 – Edge Trimming

•	RM513 – Grass and Weed Control

•	RM515 – Tree and Shrub Management

•	RM517 – Noxious Weed Control

•	RM822 – Litter Control

© VicRoads  May 2012

Section 720 (Page 16 of 16)